SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 2- DEBUGGING & OPTIMISATION TASK

| Course Code: | CSA12 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO2 | Assessment: | ASSESSMENT TOOL2- DEBUGGING & OPTIMISATION TASK |
| Weightage: | 20% | Total Marks: | 25 |
| CO2 Statement: | Apply integer and floating-point arithmetic algorithms including Booth's multiplication, restoring/non-restoring division, and IEEE 754 standard operations to solve fixed-point and floating-point computational problems. (BL3) |  |  |
| Student Name: | KV GAGAN YADAV | Reg.No.: | 192412406 |
| Department: | ECE | Date: | 1-08-2026 |

ANNEXURE A

- A program performing signed integer addition using 2’s complement produces incorrect results when adding large values (e.g., +120 and +50 in 8-bit representation). Debug the issue by analyzing the binary computation and identifying whether overflow has occurred. Propose a solution to correctly detect and handle overflow in such cases.

- A processor using a ripple carry adder shows significant delay in executing arithmetic operations in a real-time system. Debug the performance bottleneck by examining how carry propagates through each bit. Suggest an optimized solution using a carry look-ahead adder, and explain how it reduces delay.

- A multiplication unit implementing Booth’s algorithm gives incorrect results for certain negative operands. Debug the step-by-step execution of the algorithm, focusing on bit-pair checking, arithmetic shifts, and sign extension. Identify the possible source of error and suggest corrections to ensure accurate signed multiplication.

- An embedded system using the restoring division algorithm experiences inefficiency due to repeated restoration steps, leading to increased execution time. Debug the algorithm’s workflow and identify where unnecessary operations occur. Propose an optimized approach using the non-restoring division method, explaining how it improves performance.

- A scientific application using IEEE 754 single precision floating-point representation produces inaccurate results when adding very small and very large numbers. Debug the issue by analyzing exponent alignment, normalization, and rounding errors. Suggest optimization techniques, such as using double precision or algorithmic adjustments, to improve accuracy.

Code of Conduct:

I  KV GAGAN YADAV(192412406) (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student: KV GAGAN YADAV

MARKS ALLOCATION

|  | Identification of Errors / Bugs (20%) | Debugging Approach & Analysis (20%) | Correctness of Debugged Solution (25%) | Optimization Techniques Applied (20%) | Testing and Documentation (15%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Identification of Errors / Bugs | 20% | Accurately identifies all errors and issues with complete understanding of the problem | Identifies most errors with minor omissions | Identifies limited errors; misses key issues | Unable to identify errors or misunderstands the problem |
| Debugging Approach & Analysis | 20% | Uses effective debugging techniques with clear analysis and root cause identification | Applies suitable debugging methods with minor gaps | Uses basic debugging methods with limited analysis | No systematic debugging approach followed |
| Correctness of Debugged Solution | 25% | Provides completely corrected solution with accurate functionality and expected output | Provides working solution with minor errors | Partially correct solution with limited functionality | Incorrect solution or fails to resolve errors |
| Optimization Techniques Applied | 20% | Applies efficient optimization methods to improve performance, memory usage and quality | Performs optimization with noticeable improvements | Limited optimization with minimal improvements | No optimization applied or inefficient implementation |
| Testing and Documentation | 15% | Performs complete testing with proper validation and clear documentation | Provides adequate testing and documentation with minor issues | Limited testing and incomplete documentation | No proper testing or documentation provided |

1. 8-Bit Signed Integer Addition & Overflow Detection

Binary Computation Analysis

In an 8-bit signed system using 2's complement, values range from -128 (100000002) to +127 (011111112).

Let's compute +120++50:

- +120 in binary: 0111 1000

- +50 in binary: 0011 0010

0111 1000+120+0011 0010+50\hline1010 1010-86 in 2's complement

Debugging the Issue

Adding two positive numbers resulted in a negative value (1010 1010 =-86). This occurs because the sum (+170) exceeds the maximum representable signed positive value (+127), causing an arithmetic overflow into the sign bit (MSB).

Solution & Overflow Detection

Overflow in 2's complement signed addition occurs if and only if:

- Two positive numbers yield a negative result, OR

- Two negative numbers yield a positive result.

Hardware-level detection uses the carry-in (Cin) and carry-out (Cout) of the Most Significant Bit (MSB):

Overflow Flag V=Cin_MSB⊕Cout_MSB

How to Handle It:

- Hardware/Assembly: Check the Overflow Flag (V). If V=1, trigger an arithmetic exception/trap or branch to a high-precision handler.

- Software: Cast operands to a wider integer type (e.g., 16-bit) prior to addition, or check bounds explicitly before executing the addition:

C

if (a > 0 && b > INT8_MAX - a) {

/* Handle Overflow Error */

}

2. Ripple Carry Adder Bottleneck & Carry Look-Ahead Optimization

Debugging the Performance Bottleneck

A Ripple Carry Adder (RCA) cascades n Full Adders (FAs) in series. The output carry of bit i (Ci+1) acts as the input carry for bit i+1.

The worst-case delay occurs when a carry must propagate from the Least Significant Bit (A0,B0) all the way to the Most Significant Bit (An-1,Bn-1).

- Gate Delay: Each FA introduces a 2-gate delay for the carry out.

- Total Propagation Delay: TRCA=2n×tgate.

- For a 32-bit adder, this introduces a significant delay of 64×tgate, creating a timing bottleneck in real-time execution.

Carry Look-Ahead Adder (CLA) Solution

A CLA removes the sequential carry propagation dependency by calculating carries in parallel using two functions for each bit position i:

- Carry Generate (Gi): Gi=Ai⋅Bi (Generates a carry regardless of input carry)

- Carry Propagate (Pi): Pi=Ai⊕Bi (Propagates an input carry)

The carry expressions are expanded directly as functions of the primary inputs (A,B) and C0:

- C1=G0+P0C0

- C2=G1+P1G0+P1P0C0

- C3=G2+P2G1+P2P1G0+P2P1P0C0

- C4=G3+P3G2+P3P2G1+P3P2P1G0+P3P2P1P0C0

Performance Improvement

Because all carry signals are generated simultaneously through a 2-level logic circuit, the carry propagation delay becomes constant (O1 complexity) relative to bit width, drastically reducing timing delays compared to the On delay of an RCA.

3. Booth’s Multiplication Algorithm Debugging

Debugging Common Sources of Error

When Booth's algorithm yields incorrect results for negative operands, the error typically stems from three implementation bugs:

- Incorrect Sign Extension during Arithmetic Right Shift (ARS): Using a Logical Right Shift instead of an Arithmetic Right Shift clears the MSB to 0 instead of preserving the sign bit (MSB remains unchanged).

- Improper Bit-Pair Encoding (Q0,Q-1):

  - 00 → No operation.

  - 01 → Add Multiplicand (A=A+M).

  - 10 → Subtract Multiplicand (A=A-M).

  - 11 → No operation.

Misinterpreting 10 as an addition or omitting Q-1 initialization (Q-1=0) corrupts negative calculations.

- Negating Multiplicand in 2’s Complement: 

Incorrectly computing -M without properly padding bit-widths leads to truncation errors.

Correct Algorithm Step-by-Step

To multiply M=-3 (1101) by Q=+2 (0010) using 4 bits:

- M=1101, -M=0011

- Accumulator A=0000, Q=0010, Q-1=0, Count=4

| Step | A | Q | Q−1​ | Bit Pair (Q0​Q−1​) | Operation |
| --- | --- | --- | --- | --- | --- |
| Init | 0000 | 0010 | 0 | - | - |
| 1 | 0000 | 0010 | 0 | 00 | No Op → ARS |
|  | 0000 | 0001 | 0 | - | Count=3 |
| 2 | 0000 | 0001 | 0 | 10 | A=A-M (0000 + 0011 = 0011) |
|  | 0011 | 0001 | 0 | - | ARS |
|  | 0001 | 1000 | 1 | - | Count=2 |
| 3 | 0001 | 1000 | 1 | 01 | A=A+M (0001 + 1101 = 1110) |
|  | 1110 | 1000 | 1 | - | ARS |
|  | 1111 | 0100 | 0 | - | Count=1 |
| 4 | 1111 | 0100 | 0 | 00 | No Op → ARS |
|  | 1111 | 1010 | 0 | - | Count=0 |

Final Result (A,Q): 1111 1010 =-6 in 2's complement. (Correct)

4. Restoring vs. Non-Restoring Division

Debugging Restoring Division

In Restoring Division, the divisor (D) is subtracted from the high-order bits of the dividend (A) in every step (A=A-D).

- If the result is negative (A<0), the step was invalid: the quotient bit is set to 0, and an extra addition step (A=A+D) is performed to restore the previous value of A.

- Inefficiency: Up to N extra restoration additions are required for an N-bit division, leading to high variable latency and wasted clock cycles.

Non-Restoring Division Optimization

Non-restoring division eliminates the explicit restore step by combining partial restoration into the subsequent cycle:

- If A≥0:

  - Shift A,Q left.

  - Subtract D (A=A-D).

- If A<0:

  - Shift A,Q left.

  - Add D (A=A+D) instead of restoring and subtracting.

Execution Speedup Comparison:

Restoring Step (when A < 0):   [Subtract D] ---> [Restore: Add D] ---> [Shift Left] ---> [Subtract D]

Non-Restoring (when A < 0):     [Subtract D] -------------------------> [Shift Left] ---> [Add D]

By postponing restoration, Non-Restoring Division guarantees exactly 1 shift and 1 arithmetic operation per bit, decreasing average clock cycles per division operation significantly.

5. IEEE 754 Floating-Point Precision & Large/Small Value Addition

Debugging Floating-Point Inaccuracy

IEEE 754 Single Precision uses 32 bits: 1 sign bit, 8 exponent bits (bias 127), and 23 mantissa bits (24 bits of precision with implicit leading 1).

When adding a very large number X and a very small number Y (e.g., 108+10-2):

- Exponent Alignment: The exponent of Y must be matched to X. The mantissa of Y is shifted right by ΔE=EX-EY positions.

- Loss of Significance (Swallowing): If ΔE>24 bits, the mantissa of Y shifts completely past the 23-bit mantissa window. Y becomes 0 during alignment.

- Result: X+Y=X. The addition of Y is entirely lost.

Optimization Techniques

- Use Double Precision (64-bit IEEE 754):

  - Offers 52 mantissa bits (53 bits of precision). This handles larger exponent differences (ΔE≤53) before underflow occurs.

- Kahan Summation Algorithm (Software-Level Optimization):

Maintains a running compensation variable (c) to accumulate lost low-order bits:

Python

def kahan_sum(numbers):

sum = 0.0

c = 0.0  # Compensation for lost low-order bits

for x in numbers:

y = x - c

t = sum + y

c = (t - sum) - y

sum = t

return sum

- Sorted Summation:

When accumulating arrays of floating-point numbers, sort values by magnitude and perform addition from smallest to largest. This prevents smaller terms from being swallowed by larger running totals early in the computation.