# SIMATS ENGINEERING

# Department of Computer Science & Engineering

# ASSESSMENT TOOL 1- EXPERIMENTS

| Course Code: | CSA12 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO3 | Assessment: | ASSESSMENT TOOL 1- EXPERIMENTS |
| Weightage: | 40% | Total Marks: | 25 |
| CO3 Statement: | Design processor organization by constructing pipelined datapath structures, identifying hazard types (data, control, structural), and comparing superscalar techniques such as out-of-order execution, speculative execution, and simultaneous multithreading (SMT). (BL6) |  |  |
| Student Name: | KV GAGAN YADAV | Reg.No.: | 192412406 |
| Department: | ECE | Date: | 8/8/2026 |

Code of Conduct:

I KV GAGAN YADAV (192412406) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

KV GAGAN YADAV

ANNEXURE A

- Design and simulate a system that performs register transfer operations along with basic arithmetic and logic operations such as addition, subtraction, AND, and OR. Use multiple registers to demonstrate how data is transferred between them during execution, and analyze the sequence of operations and the role of the ALU in processing data.

- Create a model of a computer system using single-bus and multi-bus organization, and perform data transfer operations between registers, memory, and I/O. Compare the time taken and efficiency of both approaches, and analyze how multiple bus structures improve parallel data transfer and reduce bottlenecks.

- Simulate a 5-stage instruction pipeline (Fetch, Decode, Execute, Memory, Write-back) and execute a set of instructions. Measure the performance in terms of speedup and throughput, and compare it with a non-pipelined system to evaluate the effectiveness of pipelining.

- Develop a pipeline execution scenario where data hazards, control hazards, and structural hazards occur. Observe their impact on instruction execution, and implement techniques such as stalling, data forwarding, and branch prediction to resolve these hazards and improve performance.

- Analyze a processor supporting superscalar execution, including out-of-order and speculative execution, and extend the study to include hyper-threading and simultaneous multithreading (SMT). Evaluate how multiple instructions are executed in parallel and compare the performance improvements in terms of instruction throughput and resource utilization.

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Understanding of Concepts | 25 | Demonstrates thorough understanding and effectively integrates multiple concepts/technologies | Good understanding with proper integration of most concepts | Basic understanding; limited or partial integration | Poor understanding; unable to integrate concepts |
| Experimental Design | 30 | Well-planned, systematic implementation with optimal solution | Correct implementation with minor issues | Basic implementation with errors or inefficiencies | Incorrect or incomplete implementation |
| Use of Tools | 20 | Effective and advanced use of tools/technologies with proper configuration | Appropriate use of tools with correct execution | Limited or inconsistent use of tools | Incorrect or minimal use of tools |
| Analysis of Results | 15 | Insightful analysis with accurate interpretation and validation of results | Good analysis with reasonable interpretation | Basic analysis with limited insights | Poor or incorrect analysis of results |
| Documentation | 10 | Clear, well-structured documentation with diagrams, code, and results | Organized documentation with necessary details | Basic documentation with missing details | Poor or incomplete documentation |

# Experiment No: 1

## Title

Design and Simulation of Register Transfer Operations Using Arithmetic Logic Unit (ALU) Aim

To design and simulate a register transfer system that performs arithmetic operations (Addition and Subtraction) and logical operations (AND and OR) using multiple registers and an Arithmetic Logic Unit (ALU), and to analyze the sequence of data transfer between registers during execution.

## Objective

The objectives of this experiment are:

- To understand the concept of Register Transfer Language (RTL).

- To study the organization of processor registers.

- To perform arithmetic operations using an ALU.

- To perform logical operations using an ALU.

- To observe data movement between registers.

- To analyze the role of the ALU in executing processor instructions.

- To understand how registers cooperate during instruction execution.

## Theory

Register Transfer

A register is a small, high-speed storage element inside the CPU that temporarily stores data, instructions, or addresses during program execution.

Register Transfer refers to the movement of binary information from one register to another under the control of clock signals.

The transfer is represented using Register Transfer Language (RTL).

Example:

R2 ← R1

This means that the contents of register R1 are copied into R2.

Register transfer is one of the most fundamental operations performed inside every processor. Before any computation is carried out, data must first be transferred into appropriate registers.

## Register Transfer Language (RTL)

RTL is a symbolic notation used to describe operations occurring between registers. Examples:

R1 ← R2

R3 ← R1 + R2

R4 ← R2 AND R3 R5 ← R3 OR R4

RTL provides a clear representation of how data flows inside the processor.

## Arithmetic Logic Unit (ALU)

The Arithmetic Logic Unit (ALU) is the computational core of the processor. It performs all arithmetic and logical operations.

The ALU receives inputs from processor registers, performs the specified operation, and stores the result back into another register.

The ALU typically performs operations such as:

- Addition

- Subtraction

- Increment

- Decrement

- AND

- OR

- XOR

- NOT

- Shift operations

- Comparison operations

Without the ALU, the CPU would only be capable of storing data and would not be able to perform computations.

## Registers Used

For this experiment, four general-purpose registers are used.

| Register | Initial Value (Decimal) | Binary Representation |
| --- | --- | --- |
| R1 | 25 | 00011001 |
| R2 | 15 | 00001111 |
| R3 | 0 | 00000000 |
| R4 | 0 | 00000000 |

## Block Diagram

+	+

|	Register R1	|

+	+	+

|

|

|

+	v	+

|	|

|	ALU	|

|	|

+	+	+

|

+	v	+

|	Register R3	|

+	+

+	+

|	Register R2	|

+	+

The registers provide inputs to the ALU. The ALU performs the selected operation, and the result is stored in the destination register.

## Algorithm Step 1

Initialize all processor registers.

## Step 2

Load data into R1 and R2.

## Step 3

Transfer R1 into R3.

## Step 4

Perform Addition. R3 ← R1 + R2

## Step 5

Perform Subtraction. R4 ← R1 − R2

## Step 6

Perform Logical AND. R3 ← R1 AND R2

## Step 7

Perform Logical OR. R4 ← R1 OR R2

## Step 8

Display all register values.

## Step 9

Stop execution.

## Register Transfer Operations Operation 1 – Data Transfer

R3 ← R1

| Before | After |
| --- | --- |
| R1 = 25 | R1 = 25 |
| R3 = 0 | R3 = 25 |

## Operation 2 – Addition

RTL:

R3 ← R1 + R2

Calculation: 25 + 15 = 40

Binary: 00011001

+ 00001111

00101000

Result:

R3 = 40

## Operation 3 – Subtraction

RTL:

R4 ← R1 − R2

Calculation 25 − 15 = 10

Binary 00011001

00001111

00001010

Result R4 = 10

## Operation 4 – AND Operation

RTL

R3 ← R1 AND R2

Binary 00011001

00001111

00001001

Decimal 9 Result

R3 = 9

## Operation 5 – OR Operation

RTL

R4 ← R1 OR R2

Binary 00011001

00001111

00011111

Decimal 31 Result

R4 = 31

## Simulation Table

| Step | Operation | RTL Representation | Result |
| --- | --- | --- | --- |
| 1 | Load Data | R1=25, R2=15 | Completed |
| 2 | Transfer | R3 ← R1 | 25 |
| 3 | Addition | R3 ← R1 + R2 | 40 |
| 4 | Subtraction | R4 ← R1 − R2 | 10 |
| 5 | AND | R3 ← R1 AND R2 | 9 |
| 6 | OR | R4 ← R1 OR R2 | 31 |

## Sequence of Execution

Initialize Registers

│

▼

Load Values into Registers

│

▼

Transfer Data Between Registers

│

▼ Addition

│

▼ Subtraction

│

▼

AND Operation

│

▼

OR Operation

│

▼ Store Result

│

▼

End

## Role of the ALU

The Arithmetic Logic Unit plays a significant role in processor execution. During each instruction cycle, operands stored in registers are supplied to the ALU. Depending on the control signals generated by the control unit, the ALU performs the required arithmetic or logical operation and sends the result back to the destination register.

In this experiment:

- The ALU adds the values stored in R1 and R2.

- It subtracts the second operand from the first.

- It performs bitwise AND to identify common set bits.

- It performs bitwise OR to combine set bits from both operands.

- It updates the destination register after every operation.

Thus, the ALU serves as the computational engine of the processor, enabling efficient execution of instructions.

## Observations

- Register transfer enables fast movement of data inside the CPU.

- Arithmetic operations are executed correctly by the ALU.

- Logical operations manipulate individual bits without changing register organization.

- Intermediate results are temporarily stored in destination registers.

- Register-based operations are significantly faster than memory-based operations because registers are located inside the processor.

## Advantages

- High-speed data processing.

- Reduced memory access time.

## Limitations

- Registers have limited storage capacity.

- Large datasets cannot be stored directly in registers.

## Applications

- Microprocessors

- Embedded systems

- Digital Signal Processing (DSP)

## Conclusion

The experiment successfully demonstrated the design and simulation of a register transfer system capable of performing arithmetic and logical operations using multiple registers and an Arithmetic Logic Unit (ALU). Data transfer between registers was verified using Register Transfer Language (RTL), and operations such as addition, subtraction, AND, and OR were executed accurately. The simulation showed that the ALU acts as the computational core of the processor by performing all arithmetic and logical computations while registers provide temporary storage for operands and results. This experiment provides a clear understanding of processor organization, data flow, and the importance of register transfer operations in achieving efficient instruction execution. It forms the foundation for advanced topics such as pipelining, hazard handling, and superscalar processor architecture.

# Experiment No: 2

Title

Design and Simulation of Single-Bus and Multi-Bus Organization for Data Transfer Between Registers, Memory, and I/O Devices

Aim

To design and simulate a computer system using both single-bus and multi-bus organizations, perform data transfer operations among registers, memory, and input/output devices, compare their performance, and analyze how multiple bus structures improve processor efficiency by enabling parallel data transfer.

## Objective

The objectives of this experiment are:

- To understand the architecture of single-bus and multi-bus organizations.

- To study the movement of data between CPU registers, memory, and I/O devices.

- To compare execution time and efficiency of both bus organizations.

- To analyze bus contention and bottlenecks.

- To understand the advantages of parallel data transfer in modern processors.

## Theory

Bus Organization

A bus is a communication pathway that transfers data, addresses, and control signals between different components of a computer system. It acts as the backbone of data communication inside the processor and between the CPU, memory, and peripheral devices.

A typical bus consists of:

- Data Bus – Transfers data between components.

- Address Bus – Carries memory addresses.

- Control Bus – Carries control signals such as Read, Write, and Interrupt.

Efficient bus organization is essential because processor performance depends on how quickly data can be transferred among different hardware components.

## Single-Bus Organization

In a single-bus organization, all processor registers, memory, and I/O devices share one common communication bus. Only one data transfer can occur at any given time.

## Characteristics

- One common communication path.

- Only one transfer at a time.

- Low hardware complexity.

- Lower implementation cost.

- Increased waiting time due to bus contention.

## Working Principle

Suppose Register R1 wants to send data to Register R2 while memory simultaneously wants to send data to the CPU. Since only one bus exists, one transfer must wait until the other completes.

## Multi-Bus Organization

A multi-bus organization uses two or more buses to allow multiple transfers simultaneously. For example:

- Bus A transfers data from Register R1.

- Bus B transfers data from Register R2.

- Bus C writes results into Register R3.

Since multiple buses operate independently, several operations occur in parallel, improving processor performance.

## Block Diagram

Single-Bus Organization

+	+

|	CPU	|

| Registers/ALU |

+	+	+

|

Single System Bus

|

+	+	+

|	|

+	+	+	+

| Memory |	| I/O Device |

+-----------+	+	+

Only one communication path is available for all transfers.

## Multi-Bus Organization

+	+

|	CPU	|

| Registers and ALU	|

+	+	+	+

|	|

Bus A	Bus B

|	|

+------+	+	+

|	|

+	+	+	+

| Memory |	| I/O Device|

+	+	+	+

\

\

Bus C

|

Destination Register

Multiple buses enable simultaneous communication between different components.

## Components Used

- CPU Registers (R1, R2, R3)

- Arithmetic Logic Unit (ALU)

- Memory

- Input Device

- Output Device

- Single Bus

- Multi-Bus

- Control Unit

## Algorithm Step 1

Initialize registers, memory, and I/O devices.

## Step 2

Design a single-bus architecture.

## Step 3

Transfer data between registers.

## Step 4

Transfer data between memory and CPU.

## Step 5

Transfer data between CPU and I/O.

## Step 6

Measure execution time.

## Step 7

Design a multi-bus architecture.

## Step 8

Repeat the same transfers using multiple buses.

## Step 9

Measure execution time.

## Step 10

Compare both architectures.

## Step 11

Analyze efficiency.

## Simulation

Assume the following initial values:

| Component | Value |
| --- | --- |
| R1 | 20 |
| R2 | 35 |
| Memory Address M100 | 60 |
| Input Device | 15 |

## Single-Bus Data Transfer Operation 1

Transfer data from Memory to CPU Register. Memory → R1

## Operation 2

Transfer data from Input Device to Register. Input → R2

## Operation 3

Transfer Register to Memory. R2 → Memory

Since only one bus exists, each operation must wait until the previous operation finishes. Execution sequence:

Memory → CPU

↓

Input → CPU

↓

CPU → Memory Total transfers = 3 Execution Time

Assume one transfer requires 5 ns. Total Time

3 × 5 = 15 ns

## Multi-Bus Data Transfer

Now assume three buses are available. At the same instant,

Bus A Memory → R1 Bus B

Input → R2 Bus C

R2 → Memory

All three transfers occur simultaneously. Execution sequence

Memory → CPU

||

Input → CPU

||

CPU → Memory Execution Time

Only one transfer cycle is required. 5 ns

## Performance Comparison

| Parameter | Single Bus | Multi Bus |
| --- | --- | --- |
| Number of Buses | 1 | 3 |
| Simultaneous Transfers | No | Yes |
| Bus Contention | High | Very Low |

| Parameter | Single Bus | Multi Bus |
| --- | --- | --- |
| Parallel Execution | No | Yes |
| Data Throughput | Low | High |
| Hardware Cost | Low | High |
| Execution Time | 15 ns | 5 ns |
| Efficiency | Moderate | Excellent |

Time Analysis Single Bus Transfer 1

0 – 5 ns

Transfer 2

5 –10 ns

Transfer 3

10 –15 ns

Total 15 ns Multi Bus All transfers 0 –5 ns Total

5 ns

## Speed Improvement

Speedup is calculated as:

Speedup = Time (Single Bus) / Time (Multi Bus) Substituting the values:

Speedup = 15 / 5 = 3

## Therefore, the multi-bus organization is three times faster than the single-bus organization for the given set of transfers.

Analysis

The simulation clearly demonstrates that a single-bus organization limits communication because all devices compete for the same bus. This causes bus contention, where one data transfer must wait until another completes, increasing execution time and reducing processor efficiency.

In contrast, a multi-bus organization allows independent buses to handle different transfers simultaneously. Registers, memory, and I/O devices can communicate in parallel, significantly reducing waiting time and improving overall throughput. Although multi-bus systems require more hardware and are costlier to implement, they provide much higher performance and are therefore widely used in modern processors.

## Advantages of Single-Bus Organization

- Simple architecture.

- Easy to design and maintain.

## Limitations of Single-Bus Organization

- Only one transfer at a time.

- High bus contention.

## Advantages of Multi-Bus Organization

- Supports parallel data transfer.

- Reduces execution time.

## Limitations of Multi-Bus Organization

- Higher hardware complexity.

- Increased implementation cost.

## Applications

- Multi-core processors

- High-performance computing systems

The simulation of both bus organizations was successfully completed. The single-bus architecture executed data transfers sequentially, resulting in a total execution time of 15 ns for three transfers. The multi-bus architecture executed the same transfers in parallel, completing them in 5 ns, thereby achieving a 3× speedup. This experiment confirms that multi-bus organization significantly enhances processor performance by enabling concurrent communication, reducing bottlenecks, and improving resource utilization.

## Conclusion

This experiment successfully demonstrated the design and simulation of single-bus and multi-bus organizations for data transfer between registers, memory, and I/O devices. The comparative analysis showed that while a single-bus organization is simpler and more economical, it suffers from limited bandwidth and bus contention, leading to higher execution time. In contrast, the multi-bus organization enables simultaneous data transfers, significantly improving throughput and reducing communication delays. These observations highlight why modern processors employ multiple bus structures to support efficient parallel processing and high-speed data movement, making multi-bus organization a preferred choice for advanced computer architectures.

# Experiment No: 3

## Title

Design and Simulation of a Five-Stage Instruction Pipeline and Performance Evaluation Aim

To design and simulate a 5-stage instruction pipeline consisting of Instruction Fetch (IF), Instruction Decode (ID), Execute (EX), Memory Access (MEM), and Write-Back (WB) stages, execute a sequence of instructions, measure pipeline performance using speedup and throughput, and compare it with a non-pipelined processor.

## Objective

The objectives of this experiment are:

- To understand the concept of instruction pipelining.

- To study the five stages of instruction execution.

- To simulate the execution of multiple instructions in a pipelined processor.

- To compare pipelined and non-pipelined execution.

- To calculate speedup and throughput.

- To analyze how pipelining improves processor performance.

## Theory

Instruction Pipelining

Instruction pipelining is a processor design technique that divides instruction execution into several stages, allowing multiple instructions to be processed simultaneously. Similar to an assembly line in manufacturing, each stage performs a specific task, and different instructions occupy different stages at the same time.

Instead of waiting for one instruction to complete all stages before starting the next, pipelining overlaps the execution of instructions, significantly improving processor performance.

Although the execution time of an individual instruction does not decrease, the processor completes more instructions per unit time, increasing overall throughput.

## Five Stages of the Instruction Pipeline

- Instruction Fetch (IF)

  - The processor fetches the instruction from memory.

  - The Program Counter (PC) provides the address of the instruction.

  - After fetching, the PC is updated to point to the next instruction.

## Instruction Decode (ID)

  - The fetched instruction is decoded.

  - Source and destination registers are identified.

  - The control unit generates the required control signals.

## Execute (EX)

  - The Arithmetic Logic Unit (ALU) performs arithmetic or logical operations.

  - Effective memory addresses are calculated if required.

## Memory Access (MEM)

  - Data is read from memory (Load instruction).

  - Data is written to memory (Store instruction).

  - Arithmetic instructions usually bypass memory access.

## Write Back (WB)

  - The final result is written back into the destination register.

  - The instruction execution is completed.

## Block Diagram

+	+

| Instruction	|

| Fetch (IF)	|

+	+	+

| v

+	+

| Instruction	|

| Decode (ID)	|

+	+	+

| v

+	+

| Execute (EX) |

+	+	+

| v

+	+

| Memory Access |

| (MEM)	|

+	+	+

| v

+	+

| Write Back	|

| (WB)	|

+	+

## Instructions Used for Simulation

Assume the following instructions:

| Instruction | Operation |
| --- | --- |
| I1 | R1 ← R2 + R3 |
| I2 | R4 ← R5 − R6 |
| I3 | R7 ← Memory[200] |
| I4 | Memory[300] ← R8 |
| I5 | R9 ← R10 AND R11 |

## Algorithm Step 1

Initialize registers and memory.

## Step 2

Fetch the first instruction.

## Step 3

Decode the instruction.

## Step 4

Execute the required ALU operation.

## Step 5

Access memory if needed.

## Step 6

Write the result into the destination register.

## Step 7

Repeat the above steps for all instructions.

## Step 8

Measure execution time.

## Step 9

Calculate speedup and throughput.

## Step 10

Compare with a non-pipelined processor.

## Pipeline Execution Table

| Clock Cycle | I1 | I2 | I3 | I4 | I5 |
| --- | --- | --- | --- | --- | --- |
| 1 | IF |  |  |  |  |
| 2 | ID | IF |  |  |  |
| 3 | EX | ID | IF |  |  |
| 4 | MEM | EX | ID | IF |  |
| 5 | WB | MEM | EX | ID | IF |
| 6 |  | WB | MEM | EX | ID |
| 7 |  |  | WB | MEM | EX |
| 8 |  |  |  | WB | MEM |
| 9 |  |  |  |  | WB |

This table shows that after the pipeline is filled, multiple instructions are executed simultaneously in different stages.

## Non-Pipelined Execution

Without pipelining, every instruction completes all five stages before the next instruction begins.

| Instruction | Clock Cycles |
| --- | --- |
| I1 | 5 |
| I2 | 5 |
| I3 | 5 |
| I4 | 5 |
| I5 | 5 |

Total Clock Cycles 5 × 5 = 25 cycles

## Pipelined Execution

For a pipeline with 5 stages executing 5 instructions, the required clock cycles are: Pipeline Cycles = Number of Instructions + Number of Stages − 1

Therefore,

Pipeline Cycles = 5 + 5 − 1 = 9 cycles Performance Calculation Execution Time

Assume one clock cycle = 2 ns Non-Pipelined Processor

25 × 2 = 50 ns

## Pipelined Processor

9 × 2 = 18 ns

## Speedup

Formula

Speedup = Non-Pipelined Time / Pipelined Time Calculation

Speedup = 50 / 18

Speedup ≈ 2.78

The pipelined processor is approximately 2.78 times faster than the non-pipelined processor for the given instruction set.

## Throughput

Throughput is defined as the number of instructions completed per unit time.

## Non-Pipelined Processor

Throughput = 5 / 50

= 0.10 instructions/ns Pipelined Processor Throughput = 5 / 18

≈ 0.278 instructions/ns

The pipelined processor achieves nearly 2.8 times higher throughput than the non-pipelined processor.

## Performance Comparison

| Parameter | Non-Pipelined | Pipelined |
| --- | --- | --- |
| Number of Instructions | 5 | 5 |
| Pipeline Stages | 5 | 5 |
| Clock Cycles | 25 | 9 |
| Clock Period | 2 ns | 2 ns |
| Execution Time | 50 ns | 18 ns |
| Speedup | 1 | 2.78 |
| Throughput | 0.10 instructions/ns | 0.278 instructions/ns |
| Resource Utilization | Low | High |
| Processor Efficiency | Moderate | Excellent |

## Analysis

The simulation demonstrates that instruction pipelining significantly improves processor performance by overlapping the execution of multiple instructions. While the first instruction still requires five stages to complete, subsequent instructions enter the pipeline before previous instructions finish, ensuring continuous utilization of processor resources.

Compared to the non-pipelined processor, which executes instructions sequentially, the pipelined processor completes the same workload in only 9 clock cycles instead of 25, reducing execution time by 64%. The calculated speedup of approximately 2.78 and increased throughput highlight the effectiveness of pipelining in improving overall processor performance.

However, the ideal performance gain may be affected in practical systems due to hazards such as data dependencies, branch instructions, and resource conflicts, which introduce pipeline stalls. These challenges are addressed using advanced

techniques such as forwarding, branch prediction, and hazard detection, which are explored in subsequent experiments.

## Advantages

- Increases instruction throughput.

- Improves CPU utilization.

## Limitations

- Pipeline hazards may reduce performance.

- Increased hardware complexity.

## Applications

- Modern RISC processors (ARM, MIPS, RISC-V)

- Intel and AMD processors

## Conclusion

The experiment successfully demonstrated the operation and performance benefits of a five-stage instruction pipeline. By allowing multiple instructions to occupy different stages simultaneously, the processor achieved better resource utilization, reduced execution time, and increased throughput compared to a non-pipelined architecture. The calculated speedup validates the effectiveness of pipelining as a fundamental performance enhancement technique in modern processors. This experiment also establishes a foundation for understanding more advanced topics such as pipeline hazards, forwarding mechanisms, branch prediction, superscalar execution, and out-of-order processing, which further improve processor performance in contemporary computer architectures.

# Experiment No: 4

Title

Simulation and Analysis of Pipeline Hazards Using Stalling, Data Forwarding, and Branch Prediction

Aim

To simulate the occurrence of data hazards, control hazards, and structural hazards in a five-stage instruction pipeline, analyze their impact on processor performance, and implement hazard resolution techniques such as stalling, data forwarding, and branch prediction to improve execution efficiency.

## Objective

The objectives of this experiment are:

- To understand different types of pipeline hazards.

- To simulate data, control, and structural hazards.

- To observe how hazards affect instruction execution.

- To implement stalling, forwarding, and branch prediction techniques.

- To compare pipeline performance before and after hazard resolution.

- To evaluate the effectiveness of hazard management techniques.

## Theory

Pipeline Hazards

Instruction pipelining increases processor performance by executing multiple instructions simultaneously. However, when instructions interfere with one another, the normal flow of the pipeline is disrupted. Such situations are called pipeline hazards.

Pipeline hazards introduce delays known as pipeline stalls or bubbles, reducing processor efficiency. There are three major types of hazards:

- Data Hazard

- Control Hazard

- Structural Hazard

## Data Hazard

A data hazard occurs when an instruction depends on the result of a previous instruction that has not yet completed execution.

## Example

I1 : R1 ← R2 + R3 I2 : R4 ← R1 + R5

Instruction I2 requires the value of R1, but I1 has not yet written the updated value into the register. As a result, I2 cannot proceed immediately.

## Pipeline without Hazard Resolution

| Clock Cycle | I1 | I2 |
| --- | --- | --- |
| 1 | IF |  |
| 2 | ID | IF |
| 3 | EX | ID |
| 4 | MEM | STALL |
| 5 | WB | EX |
| 6 |  | MEM |

| Clock Cycle | I1 | I2 |
| --- | --- | --- |
| 7 |  | WB |

The processor inserts one stall cycle before I2 executes.

## Effect of Data Hazard

- Processor waits for data.

- Pipeline efficiency decreases.

- Execution time increases.

- Throughput decreases.

## Data Forwarding

Data forwarding (also called bypassing) transfers the ALU result directly from one pipeline stage to another without waiting for the Write-Back stage.

Instead of waiting,

EX → WB → Register → EX

the processor forwards the data directly:

EX → EX

## Pipeline with Forwarding

| Clock Cycle | I1 | I2 |
| --- | --- | --- |
| 1 | IF |  |
| 2 | ID | IF |
| 3 | EX | ID |
| 4 | MEM | EX |
| 5 | WB | MEM |
| 6 |  | WB |

No stall cycle is required.

## Advantages of Data Forwarding

- Eliminates unnecessary waiting.

- Improves throughput.

- Reduces execution time.

- Increases processor utilization.

## Control Hazard

A control hazard occurs due to branch or jump instructions.

When the processor encounters a branch instruction, it does not immediately know which instruction should be fetched next.

## Example

BEQ R1, R2, LABEL

The processor must first evaluate whether the branch condition is true. During this period, the pipeline may fetch incorrect instructions.

## Pipeline without Branch Prediction

| Clock Cycle | Branch | Next Instruction |
| --- | --- | --- |
| 1 | IF |  |
| 2 | ID | IF |
| 3 | EX | STALL |
| 4 | MEM | STALL |
| 5 | WB | Correct Instruction |

Two stall cycles are introduced.

## Effect of Control Hazard

- Incorrect instruction fetch.

- Pipeline flush.

- Increased execution time.

- Reduced throughput.

## Branch Prediction

Branch prediction allows the processor to predict whether the branch will be taken before the condition is fully evaluated. Common prediction methods include:

- Static Branch Prediction

- Dynamic Branch Prediction

If the prediction is correct, instruction execution continues without interruption.

## Pipeline with Branch Prediction

| Clock Cycle | Branch | Predicted Instruction |
| --- | --- | --- |
| 1 | IF |  |
| 2 | ID | IF/ |
| 3 | EX | ID |
| 4 | MEM | EX |
| 5 | WB | MEM |
| 6 |  | WB |

The pipeline executes continuously without stalls.

## Advantages of Branch Prediction

  - Reduces branch penalty.

## Structural Hazard

A structural hazard occurs when two or more instructions require the same hardware resource simultaneously.

## Example

Suppose there is only one memory unit.

Instruction I1 needs memory access. At the same time,

Instruction I2 attempts to fetch another instruction. Both require memory simultaneously.

Only one can proceed.

## Pipeline without Resolution

| Clock Cycle | I1 | I2 |
| --- | --- | --- |
| 1 | IF |  |
| 2 | ID | IF |
| 3 | EX | ID |
| 4 | MEM | STALL |
| 5 | WB | EX |
| 6 |  | MEM |
| 7 |  | WB |

A stall occurs because memory is busy. Structural Hazard Resolution Modern processors use:

- Separate Instruction Cache

- Separate Data Cache

- Multiple Memory Ports

- Multi-bus Architecture

These allow simultaneous instruction fetch and data access.

## Pipeline after Structural Hazard Resolution

| Clock Cycle | I1 | I2 |
| --- | --- | --- |
| 1 | IF |  |
| 2 | ID | IF |
| 3 | EX | ID |
| 4 | MEM | EX |
| 5 | WB | MEM |
| 6 |  | WB |

No waiting occurs because hardware resources are available.

## Simulation

Consider the following instructions: I1 : R1 ← R2 + R3

I2 : R4 ← R1 + R5

I3 : BEQ R4, R6, LABEL

I4 : R7 ← Memory[200] I5 : Memory[300] ← R8 Hazards observed:

| Instruction | Hazard Type |
| --- | --- |
| I2 depends on I1 | Data Hazard |
| I3 is Branch Instruction | Control Hazard |
| I4 and IF access memory simultaneously | Structural Hazard |

## Performance Analysis

Assume:

Pipeline without hazards 9 cycles

Pipeline with hazards 13 cycles

Pipeline after hazard resolution 10 cycles

## Speedup after Hazard Resolution

Formula

Speedup = Time Before Resolution / Time After Resolution Calculation

Speedup = 13 / 10 = 1.3

The hazard resolution techniques improve execution speed by approximately 30% compared to the unresolved pipeline.

## Comparison of Hazard Resolution Techniques

| Hazard | Cause | Resolution Technique | Improvement |
| --- | --- | --- | --- |
| Data Hazard | Instruction dependency | Data Forwarding | Eliminates unnecessary stalls |
| Control Hazard | Branch instructions | Branch Prediction | Reduces branch penalty |
| Structural Hazard | Hardware resource conflict | Separate memory/cache, multi-bus | Allows parallel resource access |

## Overall Performance Comparison

| Parameter | Without Resolution | After Resolution |
| --- | --- | --- |
| Pipeline Stalls | High | Very Low |
| Execution Cycles | 13 | 10 |
| Processor Utilization | Low | High |
| Instruction Throughput | Moderate | High |
| Pipeline Efficiency | Reduced | Improved |

## Analysis

The simulation demonstrates that pipeline hazards significantly affect processor performance by introducing delays and reducing instruction throughput. Data hazards occur due to dependencies between instructions, control hazards arise from branch instructions, and structural hazards result from competition for hardware resources. These issues interrupt the smooth flow of the pipeline and decrease overall efficiency.

By implementing data forwarding, dependent instructions receive results directly from the ALU without waiting for register updates. Branch prediction minimizes delays caused by conditional branches by predicting the execution path in advance. Improved hardware organization, such as separate instruction and data memories or multi-bus architectures, eliminates structural conflicts. Together, these techniques reduce pipeline stalls, improve resource utilization, and significantly enhance processor performance.

## Advantages

- Improves instruction throughput.

- Minimizes execution delays.

## Limitations

- Hazard detection logic increases hardware complexity.

- Incorrect branch predictions require pipeline flushing.

## Applications

- Modern Intel and AMD processors.

- ARM and RISC-V architectures.

## Conclusion

This experiment successfully analyzed the impact of pipeline hazards on processor performance and demonstrated effective techniques to overcome them. Data forwarding eliminated unnecessary waiting caused by data dependencies, branch prediction reduced delays associated with control hazards, and improved hardware organization resolved structural conflicts. The comparative performance analysis confirmed that these hazard resolution techniques substantially enhance pipeline efficiency by reducing stalls, improving throughput, and increasing processor utilization. These mechanisms are fundamental components of modern high-performance processors and are essential for achieving efficient instruction-level parallelism in contemporary computer architectures.

# Experiment No: 5

Title

Analysis and Simulation of Superscalar Processor Architecture with Out-of-Order Execution, Speculative Execution, Hyper-Threading, and Simultaneous Multithreading (SMT)

Aim

To analyze and simulate the working of a superscalar processor that supports out-of-order execution, speculative execution, hyper-threading, and simultaneous multithreading (SMT), and evaluate their impact on instruction throughput, processor utilization, and overall system performance.

## Objective

The objectives of this experiment are:

- To understand the concept of superscalar processor architecture.

- To study out-of-order execution and speculative execution.

- To understand hyper-threading and simultaneous multithreading (SMT).

- To analyze parallel instruction execution.

- To compare processor performance with scalar processors.

- To evaluate throughput and resource utilization.

## Theory

Superscalar Processor

A superscalar processor is an advanced CPU architecture capable of issuing and executing multiple instructions during a single clock cycle by using multiple execution units.

Unlike a scalar processor, which executes only one instruction per clock cycle, a superscalar processor identifies independent instructions and executes them simultaneously.

This improves processor performance without increasing the clock frequency.

## Working Principle

The processor contains multiple functional units such as:

- Integer ALUs

- Floating Point Units (FPU)

- Load/Store Units

- Branch Units

The instruction scheduler checks whether instructions are independent. If they are, they are dispatched to different execution units simultaneously.

## Block Diagram

+	+

| Instruction Fetch |

+	+	+

|

+	v	+

| Instruction Decode |

+	+	+

|

+	v	+

| Issue & Dispatch Unit|

+	+	+	+	+

|	|	|

+	+	|	+	+

|	|	|

+	v	+	+	v	+	+	v	+

| Integer ALU |	| Floating ALU|	| Load/Store |

+	+	|	+	+	|	+	+	+

|	|	|

+	+	+

|

+	v	+

| Write Back Unit	|

+	+

The dispatch unit assigns instructions to different execution units, enabling multiple instructions to execute concurrently.

## Out-of-Order Execution Definition

Out-of-order execution is a technique in which the processor executes instructions as soon as their operands become available, rather than strictly following the original program order.

Independent instructions are executed earlier while dependent instructions wait for the required data.

## Example

I1 : R1 ← R2 + R3 I2 : R4 ← R1 + R5 I3 : R6 ← R7 + R8

Here:

- I2 depends on the result of I1.

- I3 is independent. Instead of waiting:

I1 → I2 → I3

The processor executes:

I1 → I3 → I2

This keeps execution units busy and improves processor utilization.

## Advantages of Out-of-Order Execution

- Reduces processor idle time.

- Improves instruction throughput.

- Increases hardware utilization.

- Enhances execution efficiency.

## Speculative Execution Definition

Speculative execution allows the processor to execute instructions before it is certain that they are needed. The processor predicts the outcome of branch instructions using branch prediction.

If the prediction is correct, execution continues without delay.

If incorrect, speculative results are discarded, and the correct instruction path is executed.

## Example

IF (A>B)

Execute Block A ELSE

Execute Block B

The processor predicts the most likely branch and starts executing it before the branch condition is fully resolved.

## Advantages of Speculative Execution

- Minimizes branch delay.

- Improves pipeline efficiency.

## Hyper-Threading Definition

Hyper-threading is Intel's implementation of simultaneous execution of two logical threads on a single physical processor core.

Each physical core appears as two logical processors to the operating system.

When one thread is waiting for data, another thread utilizes the idle execution resources.

## Illustration

Physical Core

|

+	+	+

| Thread 1 |

| Thread 2 |

+	+

Both threads share the execution resources, resulting in better CPU utilization.

## Advantages of Hyper-Threading

- Improves CPU utilization.

- Better multitasking performance.

- Increases throughput.

- Reduces idle execution units. Simultaneous Multithreading (SMT) Definition

Simultaneous Multithreading (SMT) is an advanced processor technology that allows instructions from multiple threads to be executed simultaneously within the same clock cycle.

Unlike hyper-threading, which typically supports two threads per core, SMT can support multiple threads depending on the processor architecture.

## Working

Example:

Thread 1 → ALU 1

Thread 2 → ALU 2

Thread 3 → Load Unit

Thread 4 → Floating Point Unit

Multiple execution units process instructions from different threads concurrently.

## Difference Between Hyper-Threading and SMT

| Parameter | Hyper-Threading | SMT |
| --- | --- | --- |
| Developer | Intel | General processor architecture |
| Threads per Core | Usually 2 | 2 or more |
| Execution | Logical dual-threading | Multiple simultaneous threads |
| Hardware Utilization | High | Very High |
| Performance | Improved | Higher than Hyper-Threading |

## Simulation

Assume the processor executes the following instructions: I1 : ADD R1,R2,R3

I2 : SUB R4,R5,R6

I3 : MUL R7,R8,R9

I4 : LOAD R10,[200]

I5 : STORE R11,[300]

## Scalar Processor

Instructions executed:

Cycle 1 → I1

Cycle 2 → I2

Cycle 3 → I3

Cycle 4 → I4

Cycle 5 → I5 Execution Cycles 5 cycles

## Superscalar Processor

Two instructions execute simultaneously. Cycle 1 → I1 + I2

Cycle 2 → I3 + I4

Cycle 3 → I5 Execution Cycles 3 cycles

Performance Analysis Assume one clock cycle = 2 ns Scalar Processor

Execution Time = 5 × 2= 10 ns Superscalar Processor Execution Time = 3 × 2= 6 ns Speedup

Formula

Speedup = Scalar Time / Superscalar Time Calculation

Speedup = 10 / 6≈ 1.67

Thus, the superscalar processor is approximately 1.67 times faster than the scalar processor for the given instruction set.

## Throughput Scalar Processor

Throughput = 5 / 10= 0.50 instructions/ns

## Superscalar Processor

Throughput = 5 / 6≈ 0.83 instructions/ns

The superscalar processor achieves significantly higher throughput due to parallel instruction execution.

## Resource Utilization Comparison

| Parameter | Scalar Processor | Superscalar Processor |
| --- | --- | --- |
| Instructions per Cycle | 1 | Up to 2 or more |
| Execution Units Used | One | Multiple |
| Parallel Execution | No | Yes |
| Processor Utilization | Moderate | High |
| Throughput | Low | High |
| Execution Time | 10 ns | 6 ns |
| Overall Performance | Good | Excellent |

## Analysis

The simulation demonstrates that superscalar processors substantially improve performance by executing multiple independent instructions during the same clock cycle. Out-of-order execution prevents execution units from remaining idle by allowing independent instructions to proceed while dependent instructions wait for operands. Speculative execution minimizes branch delays by predicting likely execution paths, thereby maintaining a continuous instruction flow. Hyper-threading increases processor utilization by allowing two logical threads to share a physical core, while Simultaneous Multithreading (SMT) further enhances performance by enabling multiple threads to execute concurrently across available execution units.

The comparison with a scalar processor shows that the superscalar architecture completes the same workload in fewer clock cycles, increasing instruction throughput and making more effective use of hardware resources. These techniques are fundamental to the high performance achieved by modern multicore processors.

## Advantages

- Executes multiple instructions simultaneously.

- Improves instruction throughput.

## Limitations

- Increased hardware complexity.

- Higher power consumption.

## Applications

- Intel Core and AMD Ryzen processors.

- ARM Cortex high-performance processors.

## Conclusion

This experiment successfully demonstrated the architecture and operation of a superscalar processor along with advanced performance enhancement techniques, including out-of-order execution, speculative execution, hyper-threading, and simultaneous multithreading (SMT). The comparative analysis showed that these techniques enable multiple instructions and threads to execute concurrently, resulting in higher throughput, better resource utilization, and reduced execution time compared to a traditional scalar processor. The findings highlight that modern processors achieve superior computational efficiency not only through higher clock frequencies but also through intelligent instruction scheduling and parallel execution mechanisms, making superscalar architecture a cornerstone of contemporary high-performance computing systems.