SIMATS ENGINEERING

Department of Computer Science & Engineering ASSESSMENT TOOL 1- ANALYTICAL PROBLEM SOLVING

| Course Code: | CSA1205 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO2 | Assessment: | ASSESSMENT TOOL1- ANALYTICAL PROBLEM SOLVING |
| Weightage: | 45% | Total Marks: | 25 |
| CO2 Statement: | Apply integer and floating-point arithmetic algorithms including Booth's multiplication, restoring/non-restoring division, and IEEE 754 standard operations to solve fixed-point and floating-point computational problems. (BL3) |  |  |
| Student Name: | KV GAGAN YADAV | Reg.No.: | 192412406 |
| Department: | ECE | Date: | 28-07-2026 |

ANNEXURE A

1. A processor performs arithmetic operations on signed 8-bit integers using 2’s complement  representation. Consider two numbers: +45 and −23. Analyze in detail how the system performs  both addition and subtraction, including binary conversion, alignment of sign bits, and  intermediate steps. Determine whether overflow occurs, and explain how the processor detects  overflow and interprets the final result.

2. A high-performance computing system replaces a ripple carry adder with a 4-bit carry look ahead adder (CLA) to reduce computation delay. Given two binary inputs, analyze how  generate and propagate signals are formed and how carries are computed in advance. Compare  the time delay of CLA with ripple carry addition and justify, with reasoning, why CLA  improves speed in modern processors.

3. In a signed multiplication operation, a processor uses Booth’s algorithm to multiply two  numbers, −6 and +5. Analyze the algorithm step-by-step, including the role of the accumulator,  multiplier, and Booth bit, along with arithmetic shifts. Explain how the algorithm efficiently  handles signed numbers and reduces the number of addition/subtraction operations compared  to traditional multiplication.

4. A processor is required to perform division of two binary numbers (e.g., 13 ÷ 3) using both  restoring and non-restoring division algorithms. Analyze both methods step-by-step, showing  intermediate remainders and quotient formation. Compare the two approaches in terms of  number of steps, complexity, and efficiency, and explain why non-restoring division is often  preferred in modern systems.

5. A scientific application requires precise handling of real numbers using the IEEE 754 standard  for floating-point representation. Given a decimal number (e.g., −13.25), analyze how it is  represented in both single precision and double precision formats, including sign, exponent,  and mantissa fields. Further, explain how floating-point addition is performed between two  such numbers, highlighting issues like normalization, rounding, and precision loss.

Code of Conduct:

I KV GAGAN YADAV(192412406) (Name / Reg No) certify that this  submission is my original work and that I have adhered to the guidelines specified for this  assessment.I understand that any violation of academic integrity rules will result in  disciplinary action.

Signature of the student: KV GAGAN YADAV

MARKS ALLOCATION

|  | Problem Understanding (20%) | Method Selection (20%) | Solution Process (25%) | Accuracy of Calculation (20%) | Interpretation of Results (15%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 20% | Clearly interprets problem, identifies all parameters and requirements accurately | Understands problem with minor gaps | Partial understanding; misses key elements | Misinterprets or unable to understand problem |
| Method Selection | 20% | Selects most appropriate method/formula with strong justification | Selects correct method with minor errors | Inappropriate or partially correct method | Unable to select method |
| Solution Process | 25% | Logical, systematic steps with correct approach throughout | Mostly correct steps with minor mistakes | Disorganized steps; multiple errors | No clear process |
| Accuracy of Calculation | 20% | All calculations accurate; correct final answer | Minor calculation errors | Frequent errors; incorrect final answer | No valid calculations |
| Interpretation of Results | 15% | Clearly interprets results with units and engineering relevance | Basic interpretation with minor omissions | Limited or unclear interpretation | No interpretation |

# Question 1:

# Step 1: Convert the Decimal Numbers into Binary

### Representation of +45

Decimal number = 45

Binary of 45:

45 = 32 + 8 + 4 + 1

Therefore,

45 = 00101101

(8-bit representation)

| Decimal | Binary |
| --- | --- |
| +45 | 00101101 |

### Representation of −23

First write +23 in binary.

23 = 16 + 4 + 2 + 1

Therefore,

23 = 00010111

Now find the 2's complement.

### Step 1: Find 1's Complement

00010111

↓

11101000

### Step 2: Add 1

11101000

+00000001

11101001

Therefore,

−23 = 11101001

| Decimal | Binary |
| --- | --- |
| −23 | 11101001 |

# Step 2: Addition (+45 + (−23))

Perform binary addition.

00101101   (+45)

+     11101001   (−23)

------------------------

1 00010110

The carry generated outside the 8-bit register is ignored.

Final 8-bit result:

00010110

Convert to decimal.

00010110

=16+4+2

=22

Therefore,

45 + (−23) = 22

## Overflow Check

Overflow occurs only when

- Two positive numbers produce a negative result.

- Two negative numbers produce a positive result.

Here,

One number is positive.

The other number is negative.

Therefore,

Overflow cannot occur.

Hence,

No Overflow

## Processor Interpretation

The processor observes that the Most Significant Bit (MSB) is 0.

MSB = 0 indicates a positive number.

Binary result

00010110

represents

22

Therefore, the processor interprets the answer as

+22

# Step 3: Subtraction (+45 − (−23))

Subtracting a negative number is equivalent to adding its positive value.

Therefore,

45 − (−23)

=45 +23

### Binary Representation

+45

00101101

+23

00010111

Now perform addition.

00101101

+     00010111

----------------

01000100

### Convert Result into Decimal

01000100

=64+4

=68

Therefore,

45 − (−23)=68

## Overflow Check

Both operands are positive.

The result is also positive.

The value 68 lies within the range of an 8-bit signed integer.

Range:

−128 to +127

Since

68 <127

there is

No Overflow

## Processor Interpretation

MSB = 0

Hence the processor identifies the result as a positive number.

Binary

01000100

is interpreted as

Decimal

68

# How the Processor Detects Overflow

The processor checks the carry into the sign bit and the carry out of the sign bit.

Overflow occurs when these two carries are different.

Another simple rule is:

- Positive + Positive → Negative → Overflow

- Negative + Negative → Positive → Overflow

- Positive + Negative → Never Overflow

In this problem,

Addition:

Positive + Negative

No Overflow

Subtraction:

Positive + Positive

Positive Result

No Overflow

Hence,

Overflow does not occur in either operation.

# Final Results

| Operation | Binary Result | Decimal Result | Overflow |
| --- | --- | --- | --- |
| +45 + (−23) | 00010110 | 22 | No |
| +45 − (−23) | 01000100 | 68 | No |

Question 2:

## Problem Understanding

In digital systems, addition is one of the most frequently performed operations. A Ripple Carry Adder (RCA) performs addition by passing the carry from one stage to the next, causing a delay. To overcome this problem, modern processors use a Carry Look-Ahead Adder (CLA), which calculates all carry values simultaneously using generate and propagate signals. This significantly increases the speed of arithmetic operations.

# Step 1: Given Binary Inputs

Let the two 4-bit binary numbers be:

A = 1011

B = 0110

Initial Carry,

C₀ = 0

| Bit Position | A | B |
| --- | --- | --- |
| A₃ B₃ | 1 | 0 |
| A₂ B₂ | 0 | 1 |
| A₁ B₁ | 1 | 1 |
| A₀ B₀ | 1 | 0 |

# Step 2: Generate (G) and Propagate (P) Signals

The CLA uses two signals for every bit.

### Generate Signal (G)

A carry is generated when both input bits are 1.

Formula:

G = A × B

### Propagate Signal (P)

A carry is propagated when either input bit is 1.

Formula:

P = A ⊕ B

Now calculate each bit.

| Bit | A | B | Generate (G=A·B) | Propagate (P=A⊕B) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 0 | 0 | 1 |
| 1 | 1 | 1 | 1 | 0 |
| 2 | 0 | 1 | 0 | 1 |
| 3 | 1 | 0 | 0 | 1 |

Therefore,

G₀ = 0

G₁ = 1

G₂ = 0

G₃ = 0

and

P₀ = 1

P₁ = 0

P₂ = 1

P₃ = 1

# Step 3: Carry Computation

Unlike the Ripple Carry Adder, the CLA computes all carries in advance.

The carry equations are:

C₁ = G₀ + (P₀ × C₀)

C₂ = G₁ + (P₁ × G₀) + (P₁ × P₀ × C₀)

C₃ = G₂ + (P₂ × G₁) + (P₂ × P₁ × G₀) + (P₂ × P₁ × P₀ × C₀)

C₄ = G₃ + (P₃ × G₂) + (P₃ × P₂ × G₁) + (P₃ × P₂ × P₁ × G₀) + (P₃ × P₂ × P₁ × P₀ × C₀)

Since

C₀ = 0

Calculate each carry.

### Carry C₁

C₁

= 0 + (1 × 0)

= 0

### Carry C₂

C₂

= 1 + (0 × 0) + (0 × 1 × 0)

= 1

### Carry C₃

C₃

= 0 + (1 × 1)

= 1

### Carry C₄

C₄

= 0 + (1 × 0)

- (1 × 1 × 1)

= 1

Therefore,

| Carry | Value |
| --- | --- |
| C₀ | 0 |
| C₁ | 0 |
| C₂ | 1 |
| C₃ | 1 |
| C₄ | 1 |

# Step 4: Calculate the Sum

The sum equation is

S = P ⊕ Carry

### S₀

S₀

= P₀ ⊕ C₀

= 1 ⊕ 0

=1

### S₁

S₁

=0⊕0

=0

### S₂

S₂

=1⊕1

=0

### S₃

S₃

=1⊕1

=0

Final Sum

0001

Final Carry

1

Therefore,

Result = 10001

### Verification

Decimal value of

A =1011

=11

B =0110

=6

11+6

=17

Binary of 17

=10001

Hence,

The calculated result is correct.

# Step 5: Comparison between Ripple Carry Adder and Carry Look-Ahead Adder

| Ripple Carry Adder | Carry Look-Ahead Adder |
| --- | --- |
| Carry moves from one stage to the next. | All carries are calculated simultaneously. |
| High delay because every carry waits for the previous carry. | Very low delay because carries are generated in advance. |
| Simple hardware design. | More complex hardware design. |
| Suitable for small circuits. | Suitable for high-speed processors. |
| Slower performance. | Faster performance. |

# Step 6: Time Delay Analysis

### Ripple Carry Adder

Suppose each full adder requires 1 unit of time.

For a 4-bit adder,

Carry passes through four stages.

Total delay

=4 units

### Carry Look-Ahead Adder

The carry values are calculated simultaneously.

The approximate delay is only

≈2 units

Therefore,

CLA reduces the addition time by almost 50% compared to the Ripple Carry Adder.

# Why CLA Improves Speed in Modern Processors

The Carry Look-Ahead Adder is faster because it does not wait for the carry to travel through each full adder. Instead, it uses Generate and Propagate signals to determine all carry bits at the same time. This reduces computation delay and increases the speed of arithmetic operations. Since processors perform millions of additions every second, reducing carry delay greatly improves overall system performance. Hence, modern CPUs and high-performance computers prefer the Carry Look-Ahead Adder over the Ripple Carry Adder.

# Final Result

| Parameter | Result |
| --- | --- |
| Binary Input A | 1011 |
| Binary Input B | 0110 |
| Generate Signals | G₀=0, G₁=1, G₂=0, G₃=0 |
| Propagate Signals | P₀=1, P₁=0, P₂=1, P₃=1 |
| Final Sum | 10001 |
| Time Delay (RCA) | 4 Units |
| Time Delay (CLA) | 2 Units |
| Faster Adder | Carry Look-Ahead Adder (CLA) |

Question 3:

## Problem Understanding

Booth's Algorithm is an efficient multiplication algorithm used for multiplying signed binary numbers represented in 2's complement form. It reduces the number of addition and subtraction operations by examining two bits of the multiplier at a time. This method is widely used in modern processors because it improves multiplication speed and correctly handles both positive and negative numbers.

# Step 1: Given Numbers

Multiplicand (M) = −6

Multiplier (Q) = +5

We use 4-bit 2's complement representation.

### Convert −6 into Binary

First, write +6.

+6 = 0110

Take 1's complement:

0110 → 1001

Add 1:

1001

+0001

1010

Therefore,

−6 = 1010

### Convert +5 into Binary

+5 = 0101

### Initial Values

| Register | Value |
| --- | --- |
| Multiplicand (M) | 1010 |
| Multiplier (Q) | 0101 |
| Accumulator (A) | 0000 |
| Booth Bit (Q₋₁) | 0 |
| Number of Bits | 4 |

# Step 2: Booth's Rules

The algorithm checks the last bit of the multiplier (Q₀) and the Booth bit (Q₋₁).

| Q₀ Q₋₁ | Operation |
| --- | --- |
| 00 | No operation |
| 11 | No operation |
| 01 | Add M to A |
| 10 | Subtract M from A |

After each operation, perform one arithmetic right shift.

# Step 3: Booth's Algorithm Execution

### Iteration 1

Initial values

A = 0000

Q = 0101

Q₋₁ = 0

Q₀Q₋₁ = 10

According to Booth's Rule,

Subtract M from A

Since

M = 1010 (-6)

Subtracting M means

A = A − M

= 0 − (−6)

= +6

Binary of +6

0110

Now,

A = 0110

Perform Arithmetic Right Shift

Before Shift

A = 0110

Q = 0101

Q₋₁ =0

After Shift

A =0011

Q =0010

Q₋₁ =1

### Iteration 2

Current values

A =0011

Q =0010

Q₋₁ =1

Q₀Q₋₁

=01

Rule

Add M

A

=0011

+1010

1101

Perform Arithmetic Right Shift

After Shift

A =1110

Q =1001

Q₋₁ =0

### Iteration 3

Current values

A =1110

Q =1001

Q₋₁ =0

Q₀Q₋₁

=10

Rule

Subtract M

Subtracting −6 is equivalent to adding +6.

A

1110

+0110

0100

Ignore carry.

Perform Arithmetic Right Shift

A =0010

Q =0100

Q₋₁ =1

### Iteration 4

Current values

A =0010

Q =0100

Q₋₁ =1

Q₀Q₋₁

=01

Rule

Add M

A

0010

+1010

1100

Perform Arithmetic Right Shift

A =1110

Q =0010

Q₋₁ =0

# Step 4: Final Product

The final product is obtained by combining

A and Q.

A =1110

Q =0010

Final Product

11100010

# Step 5: Verify the Result

Decimal multiplication

−6 × 5

= −30

Now convert −30 into 8-bit binary.

+30

=00011110

1's Complement

11100001

Add 1

11100010

Therefore,

−30 = 11100010

This matches the result obtained using Booth's Algorithm.

Hence,

Answer is Correct.

# Role of Registers

### Accumulator (A)

The accumulator stores the intermediate result after each addition or subtraction.

### Multiplier (Q)

The multiplier contains the second operand and is shifted after every iteration.

### Booth Bit (Q₋₁)

The Booth bit stores the previous least significant bit of the multiplier. It helps determine whether addition, subtraction, or no operation should be performed.

# Role of Arithmetic Right Shift

After each iteration, the combined register (A, Q, Q₋₁) is shifted to the right.

The sign bit of A is preserved during the shift. This is called an Arithmetic Right Shift, and it ensures that negative numbers remain negative after shifting.

# Advantages of Booth's Algorithm

- It correctly handles both positive and negative numbers.

- It reduces the number of addition and subtraction operations.

- It is faster than ordinary binary multiplication.

- It efficiently handles consecutive 1's in the multiplier.

- It is widely used in modern processors and digital signal processors.

# Comparison with Traditional Multiplication

| Traditional Multiplication | Booth's Algorithm |
| --- | --- |
| Performs addition for every 1 in the multiplier. | Performs fewer additions and subtractions. |
| Slower for signed numbers. | Faster for signed numbers. |
| More computation time. | Less computation time. |
| Less efficient for long binary numbers. | More efficient for large binary numbers. |

# Final Result

| Parameter | Value |
| --- | --- |
| Multiplicand (M) | −6 (1010) |
| Multiplier (Q) | +5 (0101) |
| Final Binary Product | 11100010 |
| Decimal Product | −30 |

Question 4:

## Problem Understanding

Division is one of the important arithmetic operations performed by a processor. In computer architecture, Restoring Division and Non-Restoring Division are two commonly used algorithms for binary division. In this problem, the decimal numbers 13 and 3 are divided using both methods. The intermediate remainder and quotient are calculated at each step, and finally both algorithms are compared.

# Step 1: Convert Decimal Numbers into Binary

Dividend = 13

13 = 1101₂

Divisor = 3

3 = 0011₂

We use 4-bit registers.

Initial values:

| Register | Value |
| --- | --- |
| Dividend (Q) | 1101 |
| Divisor (M) | 0011 |
| Remainder (A) | 0000 |
| Number of Bits | 4 |

# PART A – Restoring Division Algorithm

## Working Principle

In the restoring division algorithm:

- Shift the remainder and quotient to the left.

- Subtract the divisor from the remainder.

- If the remainder is positive, write 1 in the quotient.

- If the remainder becomes negative, restore the previous remainder by adding the divisor again and write 0 in the quotient.

- Repeat the process until all bits are processed.

## Iteration 1

Initial values

A = 0000

Q = 1101

Shift left

A = 0001

Q = 1010

Subtract divisor

0001

−0011

=1110 (Negative)

Since the remainder is negative, restore it.

1110

+0011

=0001

Quotient bit = 0

Current values

A = 0001

Q = 1010

## Iteration 2

Shift left

A = 0011

Q = 0100

Subtract divisor

0011

−0011

=0000

Remainder is positive.

Quotient bit = 1

Current values

A = 0000

Q = 0101

Iteration 3

Shift left

A = 0000

Q = 1010

Subtract divisor

0000

−0011

=1101 (Negative)

Restore remainder

1101

+0011

=0000

Quotient bit = 0

Current values

A = 0000

Q = 1010

## Iteration 4

Shift left

A = 0001

Q = 0100

Subtract divisor

0001

−0011

=1110 (Negative)

Restore remainder

1110

+0011

=0001

Quotient bit = 0

## Final Result (Restoring Division)

The binary quotient obtained is

0100₂

Binary remainder

0001₂

Convert into decimal.

Quotient

0100₂ = 4

Remainder

0001₂ = 1

Therefore,

13 ÷ 3 = 4

Remainder = 1

# PART B – Non-Restoring Division Algorithm

## Working Principle

The non-restoring division algorithm avoids restoring the remainder whenever it becomes negative.

The steps are:

- Shift the registers to the left.

- If the previous remainder is positive, subtract the divisor.

- If the previous remainder is negative, add the divisor.

- Continue the process until all bits are completed.

- If the final remainder is negative, add the divisor once to obtain the correct remainder.

This method reduces the number of arithmetic operations.

## Iteration 1

Initial values

A = 0000

Q =1101

Shift left

A =0001

Q =1010

Subtract divisor

0001

−0011

=1110 (Negative)

Quotient bit =0

## Iteration 2

Since the previous remainder is negative,

Add divisor

1110

+0011

=0001

Shift left

A =0011

Q =0100

Subtract divisor

0011

−0011

=0000

Quotient bit =1

## Iteration 3

Previous remainder is positive.

Shift left

A =0000

Q =1010

Subtract divisor

0000

−0011

=1101 (Negative)

Quotient bit =0

## Iteration 4

Previous remainder is negative.

Add divisor

1101

+0011

=0000

Shift left

A =0001

Q =0100

Subtract divisor

0001

−0011

=1110

Quotient bit =0

Since the final remainder is negative,

Restore it once.

1110

+0011

=0001

# Final Result (Non-Restoring Division)

Quotient

0100₂

Remainder

0001₂

Decimal values

Quotient = 4

Remainder = 1

Therefore,

13 ÷ 3 = 4

Remainder = 1

# Verification

Using decimal arithmetic,

13 ÷ 3

= 4

Remainder = 1

Therefore, both restoring and non-restoring division algorithms produce the correct result.

# Comparison between Restoring and Non-Restoring Division

| Restoring Division | Non-Restoring Division |
| --- | --- |
| Restores the remainder whenever it becomes negative. | Does not restore the remainder immediately. |
| Requires more addition operations. | Requires fewer arithmetic operations. |
| Slower execution. | Faster execution. |
| Simpler to understand. | Slightly more complex but more efficient. |
| Used in basic processors. | Used in modern high-speed processors. |

# Time Complexity Comparison

| Parameter | Restoring | Non-Restoring |
| --- | --- | --- |
| Shift Operations | 4 | 4 |
| Subtractions | 4 | 4 |
| Restoring Additions | More | Fewer |
| Execution Speed | Slower | Faster |
| Hardware Efficiency | Lower | Higher |

# Why Non-Restoring Division is Preferred

The non-restoring division algorithm is preferred because it avoids restoring the remainder after every negative result. Instead, it performs the correction only when required. This reduces the number of arithmetic operations, decreases execution time, and improves processor performance. Since modern processors execute millions of arithmetic operations every second, reducing unnecessary additions helps improve speed and hardware efficiency.

# Final Result

| Algorithm | Quotient | Remainder |
| --- | --- | --- |
| Restoring Division | 0100₂ (4) | 0001₂ (1) |
| Non-Restoring Division | 0100₂ (4) | 0001₂ (1) |

# Interpretation of Results

Both Restoring Division and Non-Restoring Division correctly divided 13 by 3 and produced the same result: Quotient = 4 and Remainder = 1. In the restoring division method, the processor restores the previous remainder whenever it becomes negative, resulting in additional arithmetic operations. In contrast, the non-restoring division method delays the correction until the end, reducing the number of operations and improving execution speed. Therefore, although both algorithms produce the same output, the Non-Restoring Division Algorithm is more efficient and is widely used in modern processors because it offers faster execution and better hardware utilization.

Question 5:

## Problem Understanding

Computers cannot store decimal numbers directly in memory. Instead, they use the IEEE 754 Floating-Point Standard, which represents decimal numbers in binary format. This standard stores a floating-point number using three parts:

- Sign Bit

- Exponent

- Mantissa (Fraction)

The IEEE 754 standard provides two common formats:

- Single Precision (32-bit)

- Double Precision (64-bit)

In this problem, we will represent the decimal number −13.25 in both formats and explain how floating-point addition is performed.

# Step 1: Convert the Decimal Number into Binary

Given number:

−13.25

Ignore the negative sign first.

### Convert Integer Part (13)

13 ÷ 2

| Division | Remainder |
| --- | --- |
| 13 ÷ 2 | 1 |
| 6 ÷ 2 | 0 |
| 3 ÷ 2 | 1 |
| 1 ÷ 2 | 1 |

Reading from bottom,

13 = 1101₂

### Convert Fractional Part (0.25)

Multiply by 2 repeatedly.

| Calculation | Integer Part |
| --- | --- |
| 0.25 × 2 = 0.50 | 0 |
| 0.50 × 2 = 1.00 | 1 |

Therefore,

0.25 = 0.01₂

### Complete Binary Number

13.25

=

1101.01₂

Since the original number is negative,

−13.25 = −1101.01₂

# Step 2: Normalize the Binary Number

IEEE 754 stores numbers in normalized scientific notation.

Move the binary point until only one digit remains before it.

1101.01₂

=

1.10101 × 2³

Therefore,

- Significand = 1.10101

- Exponent = 3

# Step 3: IEEE 754 Single Precision (32-bit)

Single precision consists of:

| Field | Number of Bits |
| --- | --- |
| Sign | 1 |
| Exponent | 8 |
| Mantissa | 23 |

Total = 32 bits

### (a) Sign Bit

Since the number is negative,

Sign = 1

### (b) Exponent

Actual exponent = 3

Bias for single precision = 127

Stored exponent

=127+3

=130

Convert 130 into binary.

130

=

10000010₂

Exponent field

=

10000010

### (c) Mantissa

Normalized number

1.10101

The leading 1 is not stored.

Only the fractional part is stored.

Mantissa

=

10101000000000000000000

(remaining bits are filled with zeros)

### Final IEEE 754 Single Precision Representation

| Field | Value |
| --- | --- |
| Sign | 1 |
| Exponent | 10000010 |
| Mantissa | 10101000000000000000000 |

Final 32-bit representation

1 10000010 10101000000000000000000

# Step 4: IEEE 754 Double Precision (64-bit)

Double precision consists of:

| Field | Number of Bits |
| --- | --- |
| Sign | 1 |
| Exponent | 11 |
| Mantissa | 52 |

Total

64 bits

### (a) Sign Bit

Negative number

Sign

=

1

### (b) Exponent

Bias for double precision

=

1023

Stored exponent

1023+3

=

1026

1026 in binary

=

10000000010₂

Exponent field

=

10000000010

### (c) Mantissa

Mantissa

=

1010100000000000000000000000000000000000000000000000

(remaining bits are filled with zeros)

### Final IEEE 754 Double Precision Representation

| Field | Value |
| --- | --- |
| Sign | 1 |
| Exponent | 10000000010 |
| Mantissa | 1010100000000000000000000000000000000000000000000000 |

Final 64-bit representation

1 10000000010 1010100000000000000000000000000000000000000000000000

# Step 5: Floating-Point Addition

Consider two numbers

13.25

and

2.5

### Binary Representation

13.25

=

1101.01₂

2.5

=

10.1₂

Normalize them.

13.25

=

1.10101 ×2³

2.5

=

1.01 ×2¹

## Step 6: Equalize the Exponents

The exponents must be equal before addition.

Difference

3−1

=2

Shift the mantissa of the smaller number.

1.01 ×2¹

↓

0.0101 ×2³

Now both numbers have exponent

2³

## Step 7: Add the Mantissas

1.10101

+     0.01010

----------------

1.11111

Exponent remains

2³

## Step 8: Normalize the Result

The answer is already normalized.

Therefore,

Result

=

1.11111 ×2³

## Step 9: Convert Back to Decimal

Binary

1111.11₂

=

15.75

Therefore,

13.25+2.5

=

15.75

# Normalization

Normalization means adjusting the binary number so that only one digit appears before the binary point.

Example

1101.01₂

↓

1.10101 ×2³

Normalization improves accuracy and allows floating-point numbers to have a standard representation.

# Rounding

Sometimes the mantissa contains more bits than can be stored.

Extra bits are removed using rounding.

Common rounding method:

Round to Nearest Even

Example

1.101010111…

↓

1.101011

This reduces storage while keeping the value as accurate as possible.

Precision Loss

Floating-point numbers cannot exactly represent every decimal value.

For example,

0.1

cannot be represented exactly in binary.

Hence,

small rounding errors occur.

After performing many calculations, these small errors may accumulate and produce slight differences in the final result.

This is known as

Precision Loss.

# Comparison between Single Precision and Double Precision

| Feature | Single Precision | Double Precision |
| --- | --- | --- |
| Total Bits | 32 | 64 |
| Sign Bits | 1 | 1 |
| Exponent Bits | 8 | 11 |
| Mantissa Bits | 23 | 52 |
| Precision | Lower | Higher |
| Memory Required | Less | More |
| Accuracy | Good | Excellent |
| Speed | Faster | Slightly Slower |

# Advantages of IEEE 754 Standard

- Provides a standard method for representing floating-point numbers.

- Supports very small and very large numbers.

- Maintains good accuracy in scientific calculations.

- Reduces calculation errors using normalization and rounding.

- Widely used in modern processors, graphics systems, and scientific applications.

# Final Result

| Representation | Value |
| --- | --- |
| Decimal Number | −13.25 |
| Binary | 1101.01₂ |
| Normalized Form | 1.10101 ×2³ |
| Single Precision | 1 10000010 10101000000000000000000 |
| Double Precision | 1 10000000010 1010100000000000000000000000000000000000000000000000 |

# Interpretation of Results

The decimal number −13.25 is represented in IEEE 754 format by dividing it into three parts: sign, exponent, and mantissa. In single precision, the number occupies 32 bits, while in double precision, it occupies 64 bits, providing higher accuracy because of the larger exponent and mantissa fields. During floating-point addition, the processor first aligns the exponents, adds the mantissas, normalizes the result, and finally applies rounding if necessary. Although floating-point arithmetic is highly efficient, some decimal values cannot be represented exactly in binary, leading to small precision losses. Therefore, the IEEE 754 standard ensures a consistent, accurate, and efficient method for handling real numbers in modern computer systems.